ECEn 240 Circuits Laboratory

Lab 6 – LINE BUFFER AMP

# PRELIMINARY PROJECT OVERVIEW

Purpose

Objectives

The purpose of this laboratory is to build and implement per spec a line buffer amplifier. Quantify the buffer amps normal & maximum operating characteristics. This will be integrated with the volume and balance controls for the stereo amplifier system you built in previous labs.

- Discuss Buffer Amp Design Specifications.

- Discuss the LF347 IC specified maximum operating limits.

- Perform Gain and Frequency Response analysis with LT Spice.

- Measure actual Input Impedance, Gain, Freq and Clipping characteristics.

- Integrate into the stereo system and demonstrate its performance for pass off.

Design Specifications

Input impedance: > 10kΩ

Voltage Gain: -3, nominal

Frequency response: flat 3 dB from 20Hz to 20kHz

Maximum input signal level: 8Vpp

Power Supply: 14 VDC

Equipment List

Stereo amplifier chassis (one per team) CD player or MP3 player

ADCOM external stereo power amplifier and speakers LF347 operational amplifier and fixed resistors (from kit) Signal generator, DMM, power supply, and oscilloscope

Discussion

Although we were able to get by with no amplifier stages in the previous labs other than a power amp, there are several good reasons to include a line buffer amplifier ahead of the volume and tone controls. First, high quality stereo amplifiers present very high input impedances (~1-20MegΩ) to the external signal source components. For example, the line-in impedance of the ADCOM amplifier you use in room 426 CB is ≈ 2.2MegΩ.

This high impedance reduces loading on the output stage of the signal source. Driving the volume control directly with the line-in signal is a low class design.

Secondly, because of losses due to signal processing we would like to boost the signal level a little for our internal processing so that we have plenty of signal to drive the final amplifier stage to full output level.

Explain two benefits of Line Buffer amp in your note book.

# Analysis & Experimental Procedures

Include your line buffer schematic from LT Spice, with component values in your write-up (if you saved this schematic from Lab 6 then print – otherwise draw again.)

Also, include a hand drawn schematic diagram of the stereo system components created so far with the Buffer amplifier integrated into it - similar to Figure 2 but as a schematic diagram (not a block diagram like figure 2).

Figure 2- Block Diagram

Input Impedance & Virtual Short Analysis

We will be implementing a two channel buffer amplifier stage that meets the stated specifications. It will be connected between the line-in signal and the volume control. The Buffer Amp was designed in Lab 6 and in this lab you will take measurements to verify the circuit meets the design specifications.

First calculate your input impedance and verify that it is greater than 10kΩ. Remember that the inverting input of an op amp is a virtual ground in the inverting amplifier configuration.

Next, use the dual power supply to power the buffer amps that you constructed on your breadboard. Take careful note of the maximum voltage ratings below and give yourself at least a 20% margin.

Figure 1. Op-Amp Connections

Set the Power Supply to Series Mode and adjust each side to VDC, then turn it OFF. Next connect the three Power Supply cables to the Stereo DC Inputs.

Measure the input impedance (resistance) as seen from the Leftin

using your DMM and record the value in Table 1. This measurement should be made with the power supply (14VDC) turned off.

Did you meet the input specification?

| POWER STATE | Input Impedance |
| --- | --- |
| 14VDC pwr ON  | 11.94 kΩ |
| 14VDC OFF, vol full cw  | 2.22 MΩ |
| Buffer Thèvenin equivalent | 11.94 kΩ |

Table 1 - Input Impedance Data

Before continuing have a TA review your Table 1: data.

As seen from the Input, what is the Thèvenin equivalent of your buffer Amp circuit, (with power ON)? What is your RS? *Explain the correlation between these two values.

With the power on, the Thevenin equilvalent is around 12k Ohms, which is the same as my Rs. This is because when power is on, the virtual short applies and we get 12 Ohms, whereas we would get something much higher (2.2M Ohms) if the power was off and there was no virtual short.

Simulate/ Predict

Print a copy of the Lab 5 frequency response plot of the line buffer amplifier (from Lab 5, Segment 2. Re-draw the schematic and re-simulate if needed.) Mark your simulation with the upper and lower frequency spec limits and corner frequency. Analyze the performance and discuss if your simulation results meet the buffer specifications. Include the marked plot in your write up.

Gain Analysis

DC power: 14VDC (Set Power Supply to Series Mode with each side at 14 V) Function Gen: 1Vpp sine wave at 2 kHz, connect to left and right buffer inputs.

(Should the output be Hi Z or 50 ohms?) Oscilloscope: Measure the left and right Buffer outputs and compute the gain.

Record in Table 2. It is easier to setup and start at 2kHz than 20Hz because the scopes have a harder time with low frequencies. Repeat at 20, 200, 20k, 200k & 2 Meg Hz.

Using the data from Table 2 enter these values onto your Bode plot printout for both Left & Right channels.

Record this question and its answer in your lab notebook: Does the observed gain response match the Design Spec gain? (If not change the Rf or Rs value until it is in spec.) Show your computations for at least one frequency.

Table 2. Line Buffer Amplifier Gain Data

| Fun.Gen. @ Sine - 1Vpp | Frequency X @ | Vin | Vout | Gain |
| --- | --- | --- | --- | --- |
|  |  | measured 1Vpp (not2V) | measured | Computed Vout/ Vin |
| Right Channel | 20 Hz | 1.07V | 3.34V | -3.12 |
|  | 200 Hz | 1.07V | 3.34V | -3.12 |
|  | 2k Hz | 1.07V | 3.30V | -3.08 |
|  | 20k Hz | 1.05V | 3.26V | -3.10 |
|  | 200k Hz | 1.05V | 3.18V | -3.02 |
|  | 2Meg Hz | 1.05V | 640mV | -0.61 |
| Left Channel | 20 Hz | 1.09V | 3.42V | -3.13 |
|  | 200 Hz | 1.09V | 3.42V | -3.13 |
|  | 2kHz | 1.06V | 3.34V | -3.15 |
|  | 20k Hz | 1.05V | 3.30V | -3.14 |
|  | 200k Hz | 1.05V | 3.26V | -3.10 |
|  | 2Meg Hz | 1.05V | 680mV | -0.65 |
| Vin = 1Vpp | fc Analysis measured |  | fc =705 kHz |  |

Frequency Performance Analysis

Verify that the frequency response of your constructed circuit is within spec, (flat within 3dB from 20 Hz to 20 kHz).

Find the Corner Frequency, fc, of your amplifier. To do this, first determine your maximum Vout from Table 2. Then adjust the frequency of the input signal until the output Vout = Vout,max * 1/√2 or (.707 *Voutmax). Enter this at the bottom of Table 2 and mark your measured corner frequency on your Bode plot.

Signal Clipping Analysis

  - Measure your 14V DC rails from the + to the – terminals using the DMM.

Record this in Table 3

  - Estimate the max Vout peak-peak signal given the DC rail voltage. Refer to Figure 2.

Explain the connection between a) and b).

  - Compute the max Vin p-p signal level without signal clipping.

Simply divide the max Vout estimated by your Gain. max Vout ÷ Gain = max Vin

28V/3 = 9.33

Show your computations

  - Measure max Vin undistorted. Set function generator at 2 kHz, increase the Vin amplitude until you notice Vout distortion or clipping. Record this maximum Vin input voltage that doesn’t cause clipping and the resulting max undistorted Vout .

+14V 0V

-14V

Too much Vin or Gain can cause clipping.

Figure 3 – Clipping and Distortion

Include some graphical representation of clipping in your write up.

## Table 3. Line Buffer Amplifier Data

| F.G. Signal @ 2 kHz | DC rails a) measured | max Vout pp b) estimated | max Vin undistorted c) computed | max Vin undistorted d) measured | max Vout undistorted d) measured |
| --- | --- | --- | --- | --- | --- |
| Max Buff Input Vpp @ Max Non-Distorted Output | 28V | 28V | 8.95V | 7.62V | 25.3V |

Do your Table 3 measurements meet spec? Show that your line buffer amplifier meets spec? Compare the maximum input specification to your line buffer amp’s maximum input with an undistorted output.

Yes, the max Vin measured is within 5% of the specific 8Vpp.

Final Performance Evaluation

Solder: Once you have verified your circuit operates within spec with the resistor values you selected you will transfer those resistors from the breadboard to your PCB Daughter Board and solder them in. Do not solder the LF347, just plug it in. Solder the grounds to the non-inverting input pins per your schematic. This is done on the bottom of the daughter PCB as shown below on the right.

Integrate: a) Insert your soldered Daughter Board into the Buffer position on the Mother Board as show in Figure 3.

- Connect from the In jack on the front panel to the buffer input header for both the left and right channels.

- Connect both channels of the buffer output to the Master Volume inputs.

- Make sure the Volume output is properly connected to the Balance pot input and that the Balance output is connected to the output Header.

Figure 4 Integrated Buffer Amp

Out

d)

a)

b)

c)

d)

In

- Connect the Power Supply cables to the DC Inputs on the chassis. (The DC Inputs on the chassis are shown on the upper right in Figure 3.)

- Connect the CD player and external power amplifier to your system and evaluate its performance.

TA Pass Off

Demonstrate the following to a TA to obtain a signoff:

  - The integrated system’s performance from f) above.

  - Your drawn schematic of this integrated stereo system

  - Good, clean solder joints on your buffer amp

# Conclusion

Write a couple of paragraphs of conclusion for your lab experience. Note if you were able to meet the specifications and if you had to make any changes to meet them.

What debug and redesign procedures did you need to perform to get it to work?

In this conclusion, or elsewhere in the body of your write up, make sure you included how your circuit performed compared to the following specifications:

# Input Impedance Gain

Frequency Response Clipping (max input voltage)

Discuss any additional implications that you may have observed. Note anything that you learned unexpectedly.